What your runs show: the DSP sends all 520 samples. But the GUI side takes them more slowly than one packet every 5 ms, so it falls behind. The samples it hasn't taken yet show up at the start of your next run (221–520 appearing in run 2).

The fix: keep the 1 ms samples, but don't send each packet the moment it's built. Store the packets in DSP memory during the test, then send them one every 20 ms. The motor still stops at exactly 520 ms; the data just finishes arriving about 2 s later. Because nothing is sent faster than the GUI can take it, nothing piles up outside the DSP.

Start from your original Acu_standalone_task.c and ACU_TasksCreator.c (remove all the test 3 and test 6 additions, and the TEMP block). Keep volt_sat = MAX_VOLT; in the PID file. Then make these 4 changes, all in Acu_standalone_task.c.

1. Near the top, next to static uint32_t TimeStamp_T[6];:

#define STEP_TEST_END_MS    520u   /* motor stops at this time                      */

#define STEP_TX_GAP_TICKS    40u   /* send one stored packet every 40 x 0.5 ms = 20 ms */

#define STEP_PKT_MAX        110u   /* 520 / 5 = 104 packets, plus margin            */

#define STEP_PKT_WORDS       58u   /* one telemetry packet = words 0..57            */

static uint32_t StepPktQ[STEP_PKT_MAX][STEP_PKT_WORDS];  /* stored packets (~25 KB) */

static uint16_t StepPktWr   = 0u;   /* packets stored */

static uint16_t StepPktRd   = 0u;   /* packets sent   */

static uint16_t StepTxTicks = 0u;   /* ticks since last send */

2. In StepTest_Process(), inside the if(TestInProgress==0u) gate, next to Std_timestamp=0u;:

StepPktWr = 0u;  StepPktRd = 0u;  StepTxTicks = 0u;

3. At the end of ReplyToMotorTestStart(), replace only the last send line (keep the LoadFooter(...) line above it):

LoadFooter(LSB_Data, Address_TxBuff, DataSize);          /* existing */

if(TestID_N == 0x00E40000u)                              /* Step Test: store, send later */

{

if((TimeStamp_T[4] <= STEP_TEST_END_MS) && (StepPktWr < STEP_PKT_MAX))

{

for(i = 0u; i <= Address_TxBuff; i++)

{

StepPktQ[StepPktWr][i] = SPORT_TxBuff[i];

}

StepPktWr++;

}

}

else

{

(void) SPORT0_DMATransmitDataBuffer(&SPORT_TxBuff[0], Address_TxBuff);  /* other tests unchanged */

}

4. In DoTest(), replace the Step Test block with:

if(TestID_N == 0x00E40000u)          /* Step Test */

{

for(LoopCount=1; LoopCount<=4; LoopCount++)

{

PositionLoopSetPoint[LoopCount] = AmplitudeCommanded_M[LoopCount];

}

/* (a) stop the motor at 520 ms (runs once) */

if((Std_timestamp >= STEP_TEST_END_MS) && (Run_Motor_0_PIDAlgo == 1u))

{

Run_Motor_0_PIDAlgo = 0u;  Run_Motor_1_PIDAlgo = 0u;

Run_Motor_2_PIDAlgo = 0u;  Run_Motor_3_PIDAlgo = 0u;

Set_ACT1PWM_DutyCycle(0);  Set_ACT2PWM_DutyCycle(0);

Set_ACT3PWM_DutyCycle(0);  Set_ACT4PWM_DutyCycle(0);

}

/* (b) send one stored packet every 20 ms; end the test when all are sent */

StepTxTicks++;

if(StepTxTicks >= STEP_TX_GAP_TICKS)

{

StepTxTicks = 0u;

if(StepPktRd < StepPktWr)

{

(void)SPORT0_DMATransmitDataBuffer(&StepPktQ[StepPktRd][0], (STEP_PKT_WORDS - 1u));

StepPktRd++;

}

else if(Std_timestamp > STEP_TEST_END_MS)   /* motor off and all 104 packets sent */

{

NumberOfCycles_Motor_0 = 0u;  NumberOfCycles_Motor_1 = 0u;

NumberOfCycles_Motor_2 = 0u;  NumberOfCycles_Motor_3 = 0u;

TestInProgress = 0u;

TxData = (uint32_t)0x2322AF24u;

(void)SPORT0_DMATransmitDataBuffer(&TxData, 0x00u);

}

}

}

How it behaves:

- The motor runs at full PWM until 520 ms, then stops (part a). Sampling stays at 1 ms, exactly as before.

- Each 5 ms packet is now copied into StepPktQ instead of being sent. Only packets up to timestamp 520 are kept, which is 104 packets.

- Part b sends one stored packet every 20 ms. After the motor stops, the test stays "running" until the last packet is out (about 2.1 s after Start). Then it ends itself.

- Sine, Trapezoidal and the other tests are not touched.

How to run it:

- Switch the card off and on.

- Start the Step Test.

- Wait about 3 seconds before pressing anything in the GUI. The graph keeps filling after the motor has stopped.

- You should see timestamps 1 to 520 with no gaps, and the next run starting at 1 again.

If it still stops short:

- Ends somewhere past 220 but before 520: change STEP_TX_GAP_TICKS to 60u (30 ms) and try again.

- Ends at ~220 again no matter how big the gap is: then the GUI takes a fixed number of packets per run, not a rate. Tell me, and I'll give you the version that samples every 3 ms instead, which fits 520 ms into that many packets.

I can also update the doc to this 520 ms solution if you want.