Two corrections

- 0x2322AF24 is not the stop message to the GUI. The design doc (section 4.1.7) says 0x2311AF24 tells the FPGA to start streaming encoder data, and 0x2322AF24 tells the FPGA to stop. The AF byte is the FPGA's unit ID. The message the GUI is waiting for comes from ReplyToMotorTestStop(): TxData = 0x2300EE24 | TestID. When the GUI presses Stop, that word is 0x23E5EE24 ("ACU (EE) has finished Step Test Stop (E5)"). Our auto-stop never sends this, so the GUI never learns the test ended. That's why the graph and log break.

- Removing the gate was wrong, for the reason in the next section.

How the timing works

OSTmrCreate(..., 1, OS_OPT_TMR_PERIODIC, MyTimerFunc, ...) calls MyTimerFunc once per timer tick, which is 500 µs = 0.5 ms. The heartbeat code proves this: pin DPI_PB07 is LOW for 20 ticks and HIGH for 20 ticks, so one cycle is 40 ticks = 20 ms, which is the 50 Hz heartbeat in the design doc.

On every call (every 0.5 ms), the callback does this:

| Line in MyTimerFunc | Happens every | What it does |
| --- | --- | --- |
| if(TestInProgress==1u) → OSTaskSemPost(&StandAloneTask_TCB) + TimeOut_500us=1 | 0.5 ms | Wakes DoTest() and raises the flag "a new 0.5 ms tick has arrived" |
| TimeCounter_1ms>1u → TimeOut_1ms=1u | 2 calls = 1 ms | Raises the flag "1 ms has passed" |
| TimeCounter_2ms>3u → TimeOut_2ms=1 | 4 calls = 2 ms | Used only in Integrated mode (FCU replies) |
| TimeCounter_5ms 0–19 LOW, 20–39 HIGH | 40 calls = 20 ms | 50 Hz heartbeat pin |

About TimeOut_500us == 1: the 500us is part of the variable's name. The 1 is a yes/no value, not a time:

- 1 means the timer has just fired and DoTest hasn't handled this tick yet.

- 0 means DoTest has already handled it, so it's waiting for the next one.

The timer sets it to 1, and DoTest sets it back to 0 near the end. The u in 1u means "unsigned". It does not mean a unit of time, and 1s is not valid C, so write 1u.

The other time variables:

- Std_timestamp counts milliseconds since Start. It goes up by 1 every 1 ms at the end of DoTest, and it is the number on the GUI's x-axis.

- SampleIndex counts 0 to 4. Every 5 samples (5 ms), ReplyToMotorTestStart() sends one telemetry packet to the GUI.

- TimeValue is only the time axis for the Sine test. It goes up by 0.0005 every tick.

How the Step Test loops (and why the gate matters)

In ACU_StandAloneTask, the STEP_TEST_START case does not reset ACUCommandID to 0, unlike the other cases. So while a test runs, every pass of the while(true) loop goes through case STEP_TEST_START → MotorStepTestStart → StepTest_Process → DoTest. Inside DoTest, OSTaskSemPend waits for the next 0.5 ms post from the timer.

This means StepTest_Process runs every 0.5 ms for the whole test. The if(TestInProgress==0u) gate makes the setup run only on the first pass. Without the gate, Std_timestamp and SampleIndex were reset every 0.5 ms. They never reached 520 or 5, so the motor didn't stop and no data was sent. That's exactly what you saw.

What your latest tests show

You saw the motor's run time change as you changed the target. So the stop itself is working. What's broken is the GUI side: our stop sends the FPGA-stop word but not the 0x23E5EE24 reply and LoadMotorDataToFlash() that the GUI Stop button triggers. The GUI never gets its "test finished" signal, so it never finishes the log. That matches short tests showing nothing and long tests showing only a partial graph.

The fix: make the auto-stop do exactly what the GUI Stop button does

Leave StepTest_Process as it is in the PDF (gate in place). Don't touch ACU_PID_Algo.c, and keep the volt_sat = MAX_VOLT; line for 100% PWM. In DoTest(), replace your Step-Test block with this:

if(TestID_N == 0x00E40000u)          /* 0xE4 = Step Test */

{

for(LoopCount=1; LoopCount<=4; LoopCount++)

{

PositionLoopSetPoint[LoopCount] = AmplitudeCommanded_M[LoopCount];

}

/* ---- Auto-stop at 520 ms: same steps as GUI Stop button ---- */

if(Std_timestamp >= 520u)                 /* Std_timestamp = ms, same as GUI x-axis */

{

/* 1. same as case STEP_TEST_STOP */

TestInProgress = 0u;

TxData = (uint32_t)0x2322AF24u;       /* tell FPGA: stop encoder data */

(void)SPORT0_DMATransmitDataBuffer(&TxData, 0x00u);

/* 2. same as ReplyToMotorTestStop() */

NumberOfCycles_Motor_0 = 0u;  NumberOfCycles_Motor_1 = 0u;

NumberOfCycles_Motor_2 = 0u;  NumberOfCycles_Motor_3 = 0u;

Run_Motor_0_PIDAlgo = 0u;     Run_Motor_1_PIDAlgo = 0u;

Run_Motor_2_PIDAlgo = 0u;     Run_Motor_3_PIDAlgo = 0u;

Set_ACT1PWM_DutyCycle(0);

Set_ACT2PWM_DutyCycle(0);

Set_ACT3PWM_DutyCycle(0);

Set_ACT4PWM_DutyCycle(0);

TxData = (uint32_t)0x23E5EE24u;       /* tell GUI: Step Test Stop (0xE5) done */

(void)SPORT0_DMATransmitDataBuffer(&TxData, 0x00u);

LoadMotorDataToFlash();

}

}

A few notes on this code:

- No if(TimeOut_500us == 1u) wrapper is needed. DoTest only runs right after a 0.5 ms post, so the wrapper adds nothing.

- Don't call ReplyToMotorTestStop() directly. At this point ACU_RxPacket[0] still holds the Start packet, so it would send 0x23E4EE24 (Start) instead of 0x23E5EE24 (Stop). That's why the two replies are written out by hand.

- Clearing Run_Motor_X_PIDAlgo matters. The PID calls come after this block in the same pass of DoTest. With the flags at 0, the PID can't turn the PWM back on.

How to check it

- GUI: after 520 ms, the GUI should show the test as stopped by itself, with the graph and log going to about 520, just like when you press Stop.

- Exact timing: put an oscilloscope on a motor's PWM pin. The 100% PWM burst should last 520 ms (±1 ms). This checks the real time without relying on the GUI.

If the GUI still cuts off at about 225 after this, the firmware is behaving exactly like a manual Stop. Then the cut-off is on the GUI side, such as a record-length or buffer setting. Tell me what the scope and the GUI show and we'll take it from there.

*****

What most likely happens at ~220 ms

These are fin actuators, so their travel is limited (MAX_FIN_DEFLECTION is 15° in your definitions file). With volt_sat = MAX_VOLT; forcing 100% PWM, the motor drives the fin into its mechanical end stop within a couple of hundred milliseconds. Then this probably follows:

- The motor stalls. The current jumps, and the motor driver IC (the DRV8320 listed in the design doc) trips its over-current protection and raises its fault signal.

- The FPGA reports the fault to the DSP in its encoder packet (Fault_status, one bit per motor: 0x10, 0x20, 0x40, 0x80).

- The firmware's fault handling then slows everything down. Every PWM write checks that bit. Here's the design doc's copy of Set_ACT1PWM_DutyCycle:

- if((Fault_status & 0x10) == 0)      /* driver fault active */{    SRU(HIGH, DAI_PB20_I);          /* disable ACT1 */    Delay(100);    SRU(LOW, DAI_PB20_I);           /* re-enable ACT1 */}

Once a fault is active, every PID call runs this disable / Delay(100) / re-enable sequence, for every motor, every 0.5 ms tick. DoTest slows to a crawl, so Std_timestamp stops advancing at the normal rate, telemetry stops reaching the GUI, and TimeValue stalls too. That's why both stop methods end at the same ~220. They both depend on DoTest running on time, and at that point it isn't.

I can't see your hardware, so this is the most likely explanation, not a certainty. But it fits every result you've reported, and the checks below can confirm or rule it out.

How to confirm it

You need about two test runs:

- Look at the graph you already have, just before it ends at ~220. If the position trace goes flat (fin at its stop) and the motor current shoots up right before the data ends, that's the stall and driver fault.

- Put an oscilloscope on a motor's PWM pin and on its enable pin (DAI_PB20 for actuator 1). If the enable pin starts toggling at ~220 ms, the fault-handling code is running.

- Run the same test with the motor shaft free, not connected to the fin linkage, so nothing blocks it. If it now runs a clean 520 ms and the graph goes to 520, the end stop and stall are confirmed.

The solution

If the stall is confirmed, the fix is not in software:

- If the motor needs to spin for the full 520 ms, the setup has to let it. Test with the shaft free or with a load that doesn't hit the end stop.

- If the actuator really has to push against its end stop at 100% for 520 ms, the driver's over-current limit and the motor's stall-current rating become the limits. That's a decision for your hardware team, using the DRV8320 settings and the Faulhaber datasheet's stall current. Please don't remove the fault check in Set_ACTxPWM_DutyCycle to get around it. That check is what stops the driver and motor from being damaged.

Keep the stop block from my last message as it is. With the shaft free, it will stop at 520 and send the GUI the same stop reply the Stop button does.
*****

Code can't make the motor keep pushing at 100% after the driver trips. If the fin hits its end stop and stalls, the DRV8320 cuts the motor to protect itself. Overriding that risks burning the driver or the motor, so that part needs the mechanical or hardware fix.

Code can stop the fault from freezing the rest of the test. Right now, once a fault appears, every PWM write runs the disable / Delay(100) / re-enable sequence. That happens for every motor on every 0.5 ms tick, so DoTest slows down, the timestamps stop, and the graph ends at ~220. If the driver is reset only once per fault, DoTest keeps its 0.5 ms pace. Then the timestamps stay correct, the graph runs all the way to 520, the stop fires at exactly 520, and the GUI gets its stop reply. The motor still turns off when the driver trips, but your graph and log will show exactly when and why: position goes flat and current spikes at the fault.

Check first (5 minutes)

- Find the Delay() function in your code (it's declared in ACU_TasksCreator.c) and check how long Delay(100) takes. If it's a long loop (milliseconds), that confirms the slowdown.

- Look at your existing graph just before ~220. Check whether the position goes flat and the current jumps right before the data ends.

If both point to the fault, make this change.

The change: reset each driver only once per fault

In Set_ACT1PWM_DutyCycle, replace only the fault if block at the top. The rest of the function stays the same:

void Set_ACT1PWM_DutyCycle(uint8_t duty)

{

static uint8_t FaultResetDone_1 = 0u;

if((Fault_status & 0x10) == 0)          /* driver reports fault */

{

if(FaultResetDone_1 == 0u)          /* reset the driver only ONCE per fault */

{

SRU(HIGH, DAI_PB20_I);          /* ACT1 disable */

Delay(100);

SRU(LOW, DAI_PB20_I);           /* ACT1 enable  */

FaultResetDone_1 = 1u;

}

}

else

{

FaultResetDone_1 = 0u;              /* fault gone -> allow a reset next time */

}

/* ... the existing duty-cycle code below stays exactly as it is ... */

}

Make the same change in Set_ACT2PWM_DutyCycle, Set_ACT3PWM_DutyCycle and Set_ACT4PWM_DutyCycle. In each one, keep the fault bit and enable pin that function already uses, and give each its own flag (FaultResetDone_2, _3, _4).

Keep the auto-stop block exactly as it is. With DoTest running on time again, Std_timestamp will reach 520 on schedule.

A behaviour change to agree with your hardware team: today the code resets a faulted driver on every write, so it keeps retrying. With this change it retries once, and then the motor stays off until the fault clears. This is the safer behaviour, but it's flight hardware, so please get it reviewed.

What you should see after this change

- The shaft can turn freely: 100% PWM for the full 520 ms, then the stop, with the graph running 1 → 520.

- The fin hits its stop: the graph still runs 1 → 520, but position goes flat and current spikes at ~220, when the driver tripped. The motor stays off after that.

If you need the full 520 ms at 100% in the second case, that's the part code can't solve. It needs a load that doesn't stall the motor, or a driver and motor rated for that stall current for 520 ms.