Here is the whole code again, this time following the clock: where the time comes from, how each tick is counted, and what happens on each tick.

1. The big picture: four layers

Hardware timer interrupt (every 500 µs)          ← the "clock"

│  sets TimeOut_500us = 1, TimeOut_1ms, posts semaphore

▼

ACU_StandAloneTask()  (infinite while loop)       ← reads GUI commands

│  calls the test function again and again

▼

DoTest()                                          ← runs ONE tick of the test

│  waits for the timer, sets setpoints, counts time

▼

MotorTwoLoopPID_ControlAlgo(MotorNum)             ← computes PWM for one motor

Everything runs at the speed of the 500 µs tick. One tick is one pass through DoTest(), which means one PID update for each motor.

2. The clock: timer interrupt and flags

The timer interrupt routine is not in the pages you shared; it's in another file (probably the timer or ISR file). From how the flags are used, it does three things:

| Every | It does | Used for |
| --- | --- | --- |
| 500 µs | TimeOut_500us = 1 | "one control tick has passed" |
| 500 µs | posts the task semaphore | wakes up DoTest() |
| 1 ms (every 2nd tick) | TimeOut_1ms = 1 (or > 0) | data logging, 1 ms timestamps |

DoTest() clears these flags at the end of each tick, so each tick is counted only once.

To see the exact period, search your project for TimeOut_500us = or TimeOut_1ms. That will lead you to the ISR and its timer setup.

3. The main task loop: how a test keeps repeating

while(true)

{

if(TestInProgress==0u)   → OSTaskQPend(... BLOCKING ...)      // idle: sleep until GUI sends a packet

else                     → OSTaskQPend(... NON_BLOCKING ...)  // test running: just check, don't wait

(CRC check, copy packet into ACU_RxPacket[], read ACUCommandID)

switch(ACUCommandID) { ... }

}

When idle (TestInProgress = 0), the task sleeps until a GUI command arrives.

When a test is running (TestInProgress = 1), each loop pass goes like this:

- It checks the queue for a new GUI packet without waiting. Nothing new arrived, so ACUCommandID still holds the old value, e.g. STEP_TEST_START.

- The test-start cases do not reset ACUCommandID to 0, so the same case runs again: MotorStepTestStart() → StepTest_Process() → DoTest().

- StepTest_Process() sees TestInProgress == 1, so it skips the setup and goes straight to DoTest().

- DoTest() waits for the next 500 µs tick, runs one step, and returns.

- The loop repeats.

So the "test loop" is really the task loop running once per tick.

How a test ends:

- The GUI sends Stop. ACUCommandID changes, the stop function runs, and PWM goes to 0.

- Or the code stops itself by setting TestInProgress = 0 (Home All at 0.5 s, or your new 520 ms stop). The task then goes back to blocking wait.

4. DoTest(): what happens in one tick, in order

OSTaskSemPend(... BLOCKING ...);       // ① wait here until the timer says 500 µs passed

if(TestID_N == 0xE3) {...}             // ② Home All   – setpoints + time counting

if(TestID_N == 0xE4) {...}             //    Step       – setpoints (+ your 520 ms counter)

if(TestID_N == 0xE6) {...}             //    Sine       – setpoints from sin(), time counting

if(TestID_N == 0xE8) {...}             //    Trapezoid  – setpoints from ramp/flat, time counting

if(TestID_N == 0xEA) {...}             //    Flight sim – setpoints from latest GUI packet

if(cycles>0 && motor selected && Run flag==1)   // ③ run PID for each enabled motor

MotorTwoLoopPID_ControlAlgo(1..4);

if(TimeOut_1ms > 0)                    // ④ every 1 ms: store a data sample

UpdateMotor_TestData();

if(TimeOut_500us==1u)                  // ⑤ tick bookkeeping

{

TimeOut_500us = 0;                 //   clear tick flag

if(TimeOut_1ms > 0)

{

TimeOut_1ms = 0;

FSL_SampleUpdate++;            //   flight-sim ms counter

Std_timestamp++;               //   ms timestamp (1, 2, 3 ...)

TimeStamp_T[SampleIndex] = Std_timestamp;

SampleIndex++;

if(SampleIndex == 5)           //   5 samples = 5 ms

{

SampleIndex = 0;

ReplyToMotorTestStart(TestID_N);   // send data packet to GUI

}

}

}

The order matters for your change. Step ② (the time check) comes before step ③ (the PID). So on the tick when time runs out, the Run flags are cleared first and the PID is not called, which keeps PWM at 0.

5. All the time counters in the code

| Variable | Increases by | When | Unit | Used for |
| --- | --- | --- | --- | --- |
| TimeOut_500us | set to 1 by ISR | every 500 µs | flag | "tick happened" |
| TimeOut_1ms | set by ISR | every 1 ms | flag | logging tick |
| TimeValue | +0.0005f | each 500 µs tick | seconds | Home All stop (0.50 s), Sine timing, your 520 ms stop |
| TPZL_TimeAxis[i] | +LoopTime (0.0005) | each tick | seconds | position inside one trapezoid period |
| Std_timestamp | +1 | each 1 ms | ms | timestamp sent to GUI |
| SampleIndex | +1, wraps at 5 | each 1 ms | count | groups 5 samples per packet |
| FSL_SampleUpdate | +1 | each 1 ms | ms | flight-sim sample counter |
| STEP_TIME (constant) | – | – | seconds (0.5 ms) | velocity and integrator math in the PID |

Since 0.0005 s × 1040 ticks = 0.52 s, TimeValue > 0.52f becomes true after about 1040 ticks, or 1041 because of float rounding. That gives 520 to 520.5 ms.

6. Timeline of a Step Test with your 520 ms change

t = 0         GUI sends STEP_TEST_START

→ StepTest_Process(): read amplitudes, set Run flags = 1,

TestInProgress = 1, TimeValue = 0, Std_timestamp = 0,

send ACK 0x2311AF24, call DoTest()

t = 0.5 ms    Tick 1:    TimeValue = 0.0005  → PID runs → PWM 100% → motor starts

t = 1.0 ms    Tick 2:    TimeValue = 0.0010  → PID runs, 1st data sample stored

...

t = 5 ms      Tick 10:   5 samples → 1st data packet sent to GUI

...         (a data packet every 5 ms)

t = 520 ms    Tick 1040: TimeValue ≈ 0.5200

t ≈ 520.5 ms  Tick 1041: TimeValue > 0.52  → Run flags = 0, PWM = 0,

TestInProgress = 0, TimeValue = 0, send 0x2322AF24 to GUI

→ PID skipped this tick → motor stopped

after         Task goes back to blocking wait for the next GUI command

About 104 data packets (520 ms ÷ 5 ms) reach the GUI during the run.

7. Timing in each test type

Home All (0xE3)

- Setpoints are 0 (home position).

- Each tick adds 0.0005 to TimeValue. After > 0.50f (1000 ticks, 500 ms), the Run flags are cleared.

- The "all Run flags == 0" check then sets PWM to 0, TestInProgress to 0 and TimeValue to 0.

- Because volt_sat = MAX_VOLT is forced, the motor runs at full power for 500 ms even though the setpoint is 0. That is what you were seeing.

Step (0xE4)

- Constant setpoint from the GUI.

- Originally it had no time counter and ran until Stop. With your change it stops at 520 ms.

Sine (0xE6)

- From 0 to 20 ms (TimeValue <= 0.02f), the setpoint is 0. This is a short settling delay.

- After 20 ms, TimeValue1 = TimeValue − 0.02, and the setpoint is A × sin(2π·f·TimeValue1 + phase).

- Each motor's end time is timecheck = cycles / frequency. Once TimeValue > timecheck + 0.02, that motor's setpoint becomes 0.

- The Sine test never clears the Run flags by itself. The PID keeps running (holding 0) until the GUI sends Stop. With volt_sat forced to MAX, the PWM stays at 100% until Stop.

Trapezoid (0xE8)

- TPZL_TimeAxis[i] goes up 0.5 ms per tick. One period is 4 × SingleSlopeTime + 2 × FlatTime:

  - ramp up from 0 to +A

  - flat at +A

  - ramp down to −A

  - flat at −A

  - ramp back to 0

- Each tick on a ramp adds or subtracts AmplitudeIncrementForSlope × 0.0005.

- At the end of a period the time axis resets to 0 and the cycle count goes down by 1.

- At 0 cycles that motor's Run flag becomes 0. When all are 0, PWM goes to 0.

Flight Simulation (0xEA)

- There is no internal time. The setpoints are simply the latest values the GUI sent.

- The test runs until the GUI sends Stop.

8. Timing inside the PID (MotorTwoLoopPID_ControlAlgo)

The PID is called once per tick for each motor, so its fixed time step is STEP_TIME = 0.5 ms.

- Velocity. (position_now − position_previous) / STEP_TIME. The previous position is saved each tick in Pre_position_info[].

- Integrator. out = prev_out + 0.5 × STEP_TIME × (in + prev_in). This is trapezoidal integration over one tick.

- Correct tick timing matters. If ticks were late or skipped, the velocity and integral values would be wrong. That is why everything is tied to the timer and semaphore.

- Rest of the calculation per tick:

  - Position error.

  - Commanded velocity = Kp_pos × 2π × error, limited.

  - Velocity error.

  - Kp_rate × velocity error, plus the integrator, gives volt_op, which is limited to give volt_sat.

  - The forced line volt_sat = MAX_VOLT overrides this (test mode).

  - PWM duty and direction are set, then motor_cw_rotation() or motor_ccw_rotation() is called.

9. Data logging timing

- Every 1 ms, UpdateMotor_TestData() stores one sample per motor at [SampleIndex]: commanded position, achieved position, motor current, commanded rate and achieved rate.

- Every 5 ms (SampleIndex == 5), ReplyToMotorTestStart() packs the 5 samples, their timestamps and the 5 V / 28 V readings into one packet. It adds a CRC footer with LoadFooter() and sends it over SPORT DMA.

10. The rest of the code (no timing involved)

- Config read/write, Card ID read/write. Flash read or write with retries and verify. Config values are converted into floats in st_config_para (the PID gains, limits, etc.).

- Health packet. Sets the relays, reads all ADC channels (motor currents, supplies, sensors) and sends the values with the POST results.

- Firmware download (ADSP/FPGA). 256-byte packets go into the backup flash blocks. On stop, the image CRC is compared with the CRC from the GUI. If they match, the image is copied to the main blocks and the CRC and version are saved.

- SPI switching. Flash and ADC share the SPI bus, so every flash access has ADSP_FLASH_SPI_OPEN() before it and ADSP_ADC_SPI_OPEN() after it.

- Helpers.

  - formatchange() reinterprets GUI bits as a float.

  - Bit_Revarsal() flips bit order for flash.

  - FW_CRC_Compute() computes the firmware CRC.

  - LoadFooter() adds the packet CRC.

Summary

The timer interrupt fires every 500 µs. Each tick wakes DoTest() once. DoTest() adds 0.0005 s to TimeValue on every tick. When TimeValue passes the limit (0.50 s in Home All, 0.52 s with your change), the code clears the Run flags and sets PWM to 0, and the motor stops.