CODE 1: Fix (load this directly)

1. ACU_StandAloneTask.c: variables and prototype

volatile uint32_t steptest500ucount = 0u;

volatile uint8_t  steptestactive = 0u, stepteststoprequest = 0u;

static   uint32_t LastSampleMs = 0u;

static void StepTest_CatchUp(void);          /* put with the other prototypes */

In ACU_TasksCreator.c, declare these three as extern volatile (same names, same types):

extern volatile uint32_t steptest500ucount;

extern volatile uint8_t steptestactive, stepteststoprequest;

2. MyTimerFunc() in ACU_TasksCreator.c, inside the existing if(TestInProgress==1u) block

if(steptestactive==1u){

steptest500ucount++;

if(steptest500ucount >= 1040u){ stepteststoprequest=1u; steptestactive=0u; }

}

3. StepTest_Process(): add OS_ERR semErr; at the top, and these lines before TestInProgress=1u;

OSTaskSemSet(&StandAloneTask_TCB, 0u, &semErr);

steptest500ucount=0u; stepteststoprequest=0u; LastSampleMs=0u;

steptestactive=1u;

4. Catch-up function, placed before DoTest()

static void StepTest_CatchUp(void)

{

uint32_t nowMs = steptest500ucount / 2u;

while(LastSampleMs < nowMs)

{

UpdateMotor_TestData();

LastSampleMs++;

TimeStamp_T[SampleIndex] = LastSampleMs;

SampleIndex++;

if(SampleIndex == 5u){ SampleIndex=0u; ReplyToMotorTestStart(TestID_N); }

}

}

5. DoTest(), right after OSTaskSemPend(...)

if((stepteststoprequest==1u) && (TestID_N==0x00E40000u))

{

stepteststoprequest=0u;

Run_Motor_0_PIDAlgo=0u; Run_Motor_1_PIDAlgo=0u;

Run_Motor_2_PIDAlgo=0u; Run_Motor_3_PIDAlgo=0u;

Set_ACT1PWM_DutyCycle(0); Set_ACT2PWM_DutyCycle(0);

Set_ACT3PWM_DutyCycle(0); Set_ACT4PWM_DutyCycle(0);

StepTest_CatchUp();

TestInProgress=0u; steptest500ucount=0u; SampleIndex=0u; LastSampleMs=0u;

TxData=(uint32_t)0x2322AF24u;

(void)SPORT0_DMATransmitDataBuffer(&TxData,0x00);

return;

}

6. DoTest(), sample section

Replace the if(TimeOut_1ms > 0u) UpdateMotor_TestData(); line with:

if((TimeOut_1ms > 0u) && (TestID_N != 0x00E40000u)) { UpdateMotor_TestData(); }

Replace the 1 ms block with:

if(TimeOut_500us==1u)

{

TimeOut_500us=0u;

if(TimeOut_1ms>0u)

{

TimeOut_1ms=0;

FSL_SampleUpdate++;

if(TestID_N == 0x00E40000u) { StepTest_CatchUp(); }

else

{

Std_timestamp = Std_timestamp + 1u;

TimeStamp_T[SampleIndex] = Std_timestamp;

SampleIndex++;

if(SampleIndex==5u){ SampleIndex=0u; ReplyToMotorTestStart(TestID_N); }

}

}

}

7. ReplyToMotorTestStop(): add these lines

steptestactive=0u; stepteststoprequest=0u; steptest500ucount=0u; LastSampleMs=0u;

CODE 2: Debug code (add on top of Code 1, remove afterwards)

1. Variables, in ACU_StandAloneTask.c

volatile uint32_t DbgPass=0u, DbgMaxGap=0u, DbgLastTick=0u, DbgOverrun=0u, DbgSamples=0u, DbgPkts=0u;

static   uint32_t DbgGap = 0u;

2. StepTest_Process(): add before TestInProgress=1u;

DbgPass=0u; DbgMaxGap=0u; DbgLastTick=0u; DbgOverrun=0u; DbgSamples=0u; DbgPkts=0u;

3. StepTest_CatchUp(): add one line after LastSampleMs++; and one inside the packet if

DbgSamples++;                                   /* after LastSampleMs++; */

DbgPkts++;                                      /* inside if(SampleIndex == 5u){...} */

4. DoTest(): add after the stop block from Code 1

if(TestID_N==0x00E40000u)

{

DbgGap = steptest500ucount - DbgLastTick;

DbgLastTick = steptest500ucount;

DbgPass++;

if(DbgGap > DbgMaxGap) DbgMaxGap = DbgGap;

if(DbgGap > 2u) DbgOverrun++;

}

5. Test without the GUI. Add static uint8_t temp_done=0u; before the loop, and put this as the first thing inside while(true) in ACU_StandAloneTask():

if(temp_done==0u){ temp_done=1u;

ACU_RxPacket[0]=0x00E40000u; ACU_RxPacket[1]=0x3F800000u;

ACU_RxPacket[2]=0u; ACU_RxPacket[3]=0u; ACU_RxPacket[4]=0u; ACU_RxPacket[5]=0x0Fu;

ACUCommandID=STEP_TEST_START; MotorStepTestStart(); }

6. Read the results. Put a breakpoint on the TestInProgress=0u; line in the stop block and read these:

| Variable | Healthy value |
| --- | --- |
| DbgSamples | 520 |
| DbgPkts | 104 |
| TimeStamp_T[0..4] | rising values up to 520 |
| DbgPass | near 1040 means the loop keeps up. About 220–430 means each pass takes 1.2–2.4 ms |
| DbgMaxGap | 1–2 if on time. Larger means a pass was late |

Code 1 fixes the time axis and the stop, but it does not make the loop faster. If DbgPass is far below 1040, first switch CCES to a Release build with optimization on. If it's still low, send me the numbers.

Each step below shows the existing line before and after, so you can find the exact spot. Lines marked NEW are the fix (Code 1). Lines marked DEBUG are Code 2, which you can leave out or remove later. I haven't compiled this.

File 1: ACU_StandAloneTask.c

Step 1: variables, near the top

static uint32_t TimeStamp_T[6];                    /* existing */

static float_Data Float_Data;                      /* existing */

volatile uint32_t steptest500ucount = 0u;          /* NEW */

volatile uint8_t  steptestactive = 0u;             /* NEW */

volatile uint8_t  stepteststoprequest = 0u;        /* NEW */

static   uint32_t LastSampleMs = 0u;               /* NEW */

volatile uint32_t DbgPass=0u, DbgMaxGap=0u, DbgLastTick=0u, DbgOverrun=0u, DbgSamples=0u, DbgPkts=0u;  /* DEBUG */

static   uint32_t DbgGap = 0u;                     /* DEBUG */

Configuration st_config_para;                      /* existing */

void ACU_StandAloneTask(void)                      /* existing */

Step 2: prototype, in the prototype list

static void UpdateMotorsCurrentPosition(void);     /* existing */

static void StepTest_Process(void);                /* existing */

static void StepTest_CatchUp(void);                /* NEW */

static void ReplyToMotorTestStop(void);            /* existing */

Step 3: StepTest_Process(). Add the declaration at the top, and the other lines just before TestInProgress=1u;.

static void StepTest_Process(void)

{

float Float_Tmp;                               /* existing */

int32_t Encoder_Data=0;                        /* existing */

OS_ERR semErr;                                 /* NEW */

if(TestInProgress==0u)                         /* existing */

...

MotorID=(uint8_t)ACU_RxPacket[5]&0x000000FFu;   /* existing */

OSTaskSemSet(&StandAloneTask_TCB, 0u, &semErr); /* NEW */

steptest500ucount=0u; stepteststoprequest=0u;   /* NEW */

LastSampleMs=0u;                                /* NEW */

DbgPass=0u; DbgMaxGap=0u; DbgLastTick=0u; DbgOverrun=0u; DbgSamples=0u; DbgPkts=0u;  /* DEBUG */

steptestactive=1u;                              /* NEW */

TestInProgress=1u;                              /* existing */

TimeOut_500us=0u;                               /* existing */

Std_timestamp=0u;                               /* existing */

Step 4: new function. Put it between the end of StepTest_Process() and static void DoTest(...).

DoTest(TestID_N);                               /* existing */

}

}                                                       /* existing: end of StepTest_Process */

static void StepTest_CatchUp(void)                      /* NEW (whole function) */

{

uint32_t nowMs = steptest500ucount / 2u;

while(LastSampleMs < nowMs)

{

UpdateMotor_TestData();

LastSampleMs++;

DbgSamples++;                                   /* DEBUG */

TimeStamp_T[SampleIndex] = LastSampleMs;

SampleIndex++;

if(SampleIndex == 5u)

{

SampleIndex = 0u;

DbgPkts++;                                  /* DEBUG */

ReplyToMotorTestStart(TestID_N);

}

}

}

static void DoTest(uint32_t TestID)                     /* existing */

Step 5: stop block in DoTest(). It goes right after OSTaskSemPend.

OSTaskSemPend(0, OS_OPT_PEND_BLOCKING,&p_ts,&err);  /* existing */

if((stepteststoprequest==1u) && (TestID_N==0x00E40000u))   /* NEW (whole block) */

{

stepteststoprequest=0u;

Run_Motor_0_PIDAlgo=0u; Run_Motor_1_PIDAlgo=0u;

Run_Motor_2_PIDAlgo=0u; Run_Motor_3_PIDAlgo=0u;

Set_ACT1PWM_DutyCycle(0); Set_ACT2PWM_DutyCycle(0);

Set_ACT3PWM_DutyCycle(0); Set_ACT4PWM_DutyCycle(0);

StepTest_CatchUp();

TestInProgress=0u; steptest500ucount=0u; SampleIndex=0u; LastSampleMs=0u;

TxData=(uint32_t)0x2322AF24u;

(void)SPORT0_DMATransmitDataBuffer(&TxData,0x00);

return;

}

if(TestID_N==0x00E40000u)                           /* DEBUG (whole block) */

{

DbgGap = steptest500ucount - DbgLastTick;

DbgLastTick = steptest500ucount;

DbgPass++;

if(DbgGap > DbgMaxGap) DbgMaxGap = DbgGap;

if(DbgGap > 2u) DbgOverrun++;

}

/* Check for Step Test ... */                       /* existing comment */

if(TestID_N == 0x00E30000u)                         /* existing: Home All */

Step 6: sample section at the end of DoTest(). Change the two places below.

if((TimeOut_1ms > 0u) && (TestID_N != 0x00E40000u))   /* CHANGED: was if(TimeOut_1ms > 0u) */

{

UpdateMotor_TestData();                           /* existing */

}

/* Check for 05ms time is completed ... */            /* existing comment */

if(TimeOut_500us==1u)                                 /* existing */

{

TimeOut_500us=0u;                                 /* existing */

if(TimeOut_1ms>0u)                                /* existing */

{

TimeOut_1ms=0;                                /* existing */

FSL_SampleUpdate++;                           /* existing */

if(TestID_N == 0x00E40000u)                   /* NEW: wrap the old 5 lines in this if/else */

{

StepTest_CatchUp();

}

else

{

Std_timestamp=Std_timestamp + 1u;         /* existing, moved into else */

TimeStamp_T[SampleIndex] = Std_timestamp; /* existing, moved into else */

SampleIndex++;                            /* existing, moved into else */

if(SampleIndex==5u)                       /* existing, moved into else */

{ SampleIndex=0u; ReplyToMotorTestStart(TestID_N); }

}

}

}

} /* End of void DoTest(void) */                          /* existing */

Step 7: ReplyToMotorTestStop()

Set_ACT4PWM_DutyCycle(0);                             /* existing */

steptestactive=0u; stepteststoprequest=0u;            /* NEW */

steptest500ucount=0u; LastSampleMs=0u;                /* NEW */

if(TestID==0x00EB0000u){ Stop_FlightSimulationTest=1; }   /* existing */

Step 8 (temporary, no-GUI test only): ACU_StandAloneTask()

static uint16_t GUI_WordCount;                        /* existing */

static uint8_t temp_done = 0u;                        /* TEMP */

while(true)                                           /* existing */

{

if(temp_done==0u){ temp_done=1u;                  /* TEMP (whole block) */

ACU_RxPacket[0]=0x00E40000u; ACU_RxPacket[1]=0x3F800000u;

ACU_RxPacket[2]=0u; ACU_RxPacket[3]=0u; ACU_RxPacket[4]=0u; ACU_RxPacket[5]=0x0Fu;

ACUCommandID=STEP_TEST_START; MotorStepTestStart(); }

if(TestInProgress==0u)                            /* existing */

Delete this block before you use the GUI.

File 2: ACU_TasksCreator.c

Step 9: externs, near the top. Put them next to the other extern lines (for example TestInProgress).

extern volatile uint16_t TestInProgress;                  /* existing */

extern volatile uint32_t steptest500ucount;               /* NEW */

extern volatile uint8_t  steptestactive;                  /* NEW */

extern volatile uint8_t  stepteststoprequest;             /* NEW */

Step 10: MyTimerFunc(). Insert inside the if(TestInProgress==1u) block, after the 500 µs flag line.

if(TestInProgress==1u)                                /* existing */

{

OSTaskSemPost(&StandAloneTask_TCB, OS_OPT_POST_NONE, &err);   /* existing */

TimeCounter_500us=0;                              /* existing */

TimeOut_500us=1;  /* Indicates 500us is completed */          /* existing */

if(steptestactive==1u)                            /* NEW (whole block) */

{

steptest500ucount++;

if(steptest500ucount >= 1040u)

{ stepteststoprequest=1u; steptestactive=0u; }

}

}                                                     /* existing: closing brace */

The 500 µs flag line is partly blacked out in the scan, so check it against your actual file.

Read the debug results

Build, run, and put a breakpoint on the TestInProgress=0u; line in the Step 5 stop block. Read these:

- DbgSamples should be 520, and DbgPkts should be 104.

- TimeStamp_T[0..4] should show rising values up to 520.

- DbgPass near 1040 means the loop keeps up. A value of about 220–430 means each pass takes 1.2–2.4 ms.

If DbgPass is far below 1040, switch to a Release build with optimization on and test again. If it's still low, send me the numbers.

The two sets are separate. Use the NEW lines for the direct dump and the DEBUG and TEMP lines only when you want to measure.

Direct dump (for the GUI and the motor): NEW lines only

| Step | File | What to do |
| --- | --- | --- |
| 1 | ACU_StandAloneTask.c | Add the 4 NEW variables. Skip the two DEBUG lines. |
| 2 | ACU_StandAloneTask.c | Add the StepTest_CatchUp prototype. |
| 3 | ACU_StandAloneTask.c | In StepTest_Process(), add OS_ERR semErr; and the NEW lines. Skip the DEBUG line. |
| 4 | ACU_StandAloneTask.c | Add the StepTest_CatchUp() function. Skip DbgSamples++; and DbgPkts++;. |
| 5 | ACU_StandAloneTask.c | Add the stop block in DoTest(). Skip the second block marked DEBUG. |
| 6 | ACU_StandAloneTask.c | Make the two sample-section changes at the end of DoTest(). |
| 7 | ACU_StandAloneTask.c | Add the 2 NEW lines in ReplyToMotorTestStop(). |
| 9 | ACU_TasksCreator.c | Add the 3 extern lines. |
| 10 | ACU_TasksCreator.c | Add the NEW block in MyTimerFunc(). |

Do not add Step 8. It exists only so you can run a test from CCES without the GUI. If it stays in, the motor will start by itself at power-up.

Debug (only when testing in CCES without the GUI)

Add these on top of the direct dump:

- Every line marked DEBUG: the Dbg variables, the reset line in StepTest_Process(), DbgSamples++; and DbgPkts++; in StepTest_CatchUp(), and the second block in DoTest().

- Step 8, the TEMP block in ACU_StandAloneTask(), which starts a fake step command.

Then breakpoint on TestInProgress=0u; in the stop block and read DbgSamples, DbgPkts, DbgPass, DbgMaxGap and TimeStamp_T[].

Suggested order

- Add the direct-dump lines, then the debug lines and Step 8.

- Test in CCES and check the numbers (DbgSamples = 520, and so on).

- If they look right, delete Step 8 and every DEBUG line.

- Rebuild and run from the GUI.

You can also skip the debugging and go straight to the direct dump. The only downside is that if something is wrong, you will have no numbers to look at.