Good, we're making progress — the stop mechanism itself works. Now we're dealing with a timing problem, not a logic problem. Here's what's going on.

The likely root cause: TimeOut_500us never gets cleared

Look carefully at your added block and every other place in the file that reads this flag (Sine, TPZL, the Std_timestamp counter at the bottom of DoTest). Not one of them ever writes TimeOut_500us = 0u; after processing it. It is only ever cleared at test start (MotorStepTestStart, SineTest_Process, etc.).

That means the flag is intended to be a level flag: an ISR sets it to 1 every real 500 µs tick, and the consumer is supposed to clear it back to 0 after handling. Since no consumer clears it, whenever DoTest() runs faster than the ISR fires (which it does — the RTOS task-loop runs whenever the scheduler picks it, not on a strict 500 µs cadence), your block sees TimeOut_500us == 1u on multiple back-to-back task iterations for the same real tick, and adds 0.0005f each time.

That is why:

- Case 1, 2: stops at ~220 ms → your TimeValue reached 0.52 after roughly 2.36× as many increments as there were real 500 µs ticks → oversampling of about 2× per tick.

- Case 3: stops at ~235 ms → same, oversampling ~2.2×.

- Case 4: 236 → 520 continuation is because you did not power-cycle. TimeValue was already at some value from run 3 and did not reset (see below), so run 4 finished the leftover then a fresh session started at 1.

The same double/triple counting also inflates Std_timestamp, which is why your "timestamp = 220" is not 220 ms of wall clock — it is more like 90–110 ms of actual time (the timestamps 1 → 220 are not milliseconds, they are just "how many times the block ran"). That is also probably what case 5 shows: threshold 100 ms in the code, but 2030 samples accumulated because Std_timestamp inside that broken 1 ms block is being incremented every task loop, not every millisecond.

The fix — one added line

Clear the flag as soon as you consume it:

if(TimeOut_500us == 1u)

{

TimeOut_500us = 0u;                 /* <-- ADD THIS FIRST LINE */

TimeValue = TimeValue + 0.0005f;

if(TimeValue > 0.52f)

{

Run_Motor_0_PIDAlgo   = 0u;

Run_Motor_1_PIDAlgo   = 0u;

Run_Motor_2_PIDAlgo   = 0u;

Run_Motor_3_PIDAlgo   = 0u;

NumberOfCycles_Motor_0 = 0u;

NumberOfCycles_Motor_1 = 0u;

NumberOfCycles_Motor_2 = 0u;

NumberOfCycles_Motor_3 = 0u;

Set_ACT1PWM_DutyCycle(0);

Set_ACT2PWM_DutyCycle(0);

Set_ACT3PWM_DutyCycle(0);

Set_ACT4PWM_DutyCycle(0);

TestInProgress = 0u;

TimeValue      = 0.0f;

TxData = (uint32_t)0x2322AF24u;

(void)SPORT0_DMATransmitDataBuffer(&TxData, 0x00u);

}

}

Now each real 500 µs tick contributes exactly one 0.0005 increment. 1040 ticks × 500 µs = 520 ms, on the nose.

Note: this changes behavior only inside your Step-Test branch. Sine/TPZL tests are untouched — they will keep their existing (arguably buggy but working-for-you) behavior.

If it still isn't 520 ms after adding the clear

Then the ISR itself is not firing every 500 µs, and the whole file's assumption is wrong. To verify empirically, add this one-time calibration:

- Comment out the entire if(TimeValue > 0.52f) body except for Set_ACTxPWM_DutyCycle(0) calls.

- Run Step Test with a stopwatch, and when you visually stop it via the GUI, note the Std_timestamp value that came back (the GUI shows the array TimeStamp_T[]).

- If stopwatch says 5.0 s and last timestamp is 5000 → each timestamp = 1 ms → the clear worked, use TimeValue > 0.520f.

- If stopwatch says 5.0 s and last timestamp is 10000 → each timestamp = 500 µs → the 1 ms block above is also being oversampled, and you should either fix that block too or switch to counting directly:

static uint32_t StepStopTicks = 0;

if(TimeOut_500us == 1u)

{

TimeOut_500us = 0u;

StepStopTicks++;

if(StepStopTicks >= 1040u)        /* 1040 × 500us = 520ms */

{

StepStopTicks = 0u;

/* ... stop code ... */

}

}

Using an integer counter avoids float rounding altogether. Also don't forget to zero StepStopTicks = 0u; in StepTest_Process() alongside the TimeValue = 0.0f; line.

About the Faulhaber SA minimotor

The motor itself isn't the cause here — this is pure firmware timing. But: at 100 % PWM these small brushless motors can draw 5–8× rated current until PID catches up. Two things to watch during your next run:

- Confirm your bench supply is not current-limiting (it will droop the rail, feed the ADC junk, and can throw off the TimeOut_500us ISR if a brownout blip resets a timer register).

- If the motor snaps hard against the fin stop before 520 ms elapses, add a hard-stop safety: whenever |MotorCurrent_x| > SAFE_CURRENT_LIMIT for more than N ticks, cut PWM regardless of the time counter. This protects the driver stage from staying in stall.

Short answer to "why 220 ms instead of 520 ms"

TimeValue += 0.0005f was running on average ~2.36 times per real 500 µs tick, because the level-flag TimeOut_500us is not being cleared and your task iterates faster than the tick. Add TimeOut_500us = 0u; as the first line inside the if(TimeOut_500us == 1u) block, retest, and it should land at 520 ms.

Honest answer: I am reasoning from the code I can see, not from measurements. Some of it is solid, some of it is my best inference.

Let me separate what's what.

What I am sure of

- Your original TimeValue = 0.0f; line was definitely a bug. That one I can prove on paper — resetting then adding 0.0005 will mathematically never exceed 0.52. Not a guess.

- TimeOut_500us is never cleared in any consumer in the file. I grep-read all the places that reference it in the PDF; MotorStepTestStart and the test-start/stop paths only ever write 0 to it at init, never after processing. So if it is a level flag (set=1, held=1 until consumer writes 0), it will absolutely be over-processed. That's a code fact.

- The 2.36× ratio (520 ms expected / 220 ms actual) matches a double-processing hypothesis very cleanly. That's arithmetic, not a guess.

What I am inferring (educated guess)

- That TimeOut_500us is a level flag that must be cleared by the consumer. I don't have the ISR code, so I can't prove this. It could instead be an edge flag that the ISR sets to 1 for exactly one DoTest iteration and then auto-clears — in which case my fix wouldn't help, and the real problem would be somewhere else (ISR period, scheduler rate, or a timer configured wrong).

- That DoTest is being called faster than once per 500 µs. I don't see the task loop that re-invokes it. There is clearly some mechanism re-entering DoTest (otherwise the Step Test would run for one tick and quit), but I can't see it in the PDF, so I'm reasoning backwards from the observed timing.

- Case 5 (100 ms set, 2030 timestamps). This one I genuinely can't explain cleanly with my hypothesis. 100 ms threshold at any over-sampling rate should give you Std_timestamp ≈ 200 at stop, not 2030. I waved past this in my last reply because I don't have a clean story for it. Flagging it now instead of pretending.

How to turn my guess into a certainty — one measurement

Before changing any more code, do this once:

- Keep your current code (with my clear-the-flag line, or without — doesn't matter for this test).

- Replace the stop condition temporarily so it stops at an integer count of ticks instead of a float threshold:

static uint32_t DebugTicks = 0;

if(TimeOut_500us == 1u)

{

/* no clear, no clear — do NOT add TimeOut_500us = 0u for this test */

DebugTicks++;

if(DebugTicks >= 1000u)   /* should be exactly 500 ms if ISR is truly 500us */

{

DebugTicks = 0;

/* ... your stop code ... */

}

}

Time it with a stopwatch or scope the PWM pin. The number you measure tells you everything:

| Wall-clock to stop | What it means |
| --- | --- |
| ~500 ms | ISR is 500 µs and DoTest runs once per tick → my hypothesis is wrong, the original bug was only the TimeValue = 0.0f; reset |
| ~250 ms | DoTest runs 2× per real tick (flag not cleared, task loop faster than ISR) → my hypothesis is right, add the clear |
| ~100–200 ms | DoTest runs 3–5× per tick (same cause, worse) |
| ~1000 ms | ISR actually fires every 1 ms, not 500 µs → the whole file misnames the flag |

Then repeat with TimeOut_500us = 0u; added as the first line in the block. If the time doubles toward 500 ms after adding the clear, we have proof.

So to answer directly

Guessing part: which specific mechanism makes it run 2.36× fast.
Not guessing: that your reset-to-zero line was broken, and that nothing in the file clears TimeOut_500us.

The one-line fix (TimeOut_500us = 0u; at the top of the block) is cheap and safe to try — if it is the problem, you're done; if it isn't, the measurement above will tell us where to look next (probably the ISR configuration or the task scheduler period). Would be a 10-minute scope session to settle it for good.